# The Arctic Lens — What We Changed Today (easy + detailed)

This explains, in simple words, everything we changed in the project today. For each change: what it was before, what it is now, and why the new way is better. All files are in D:\DESKTOP\lab 2\ESAME - Arctic Lens\.

(Small words explained: void = an empty open cell with no skin · pod = one truncated-octahedron cell · cluster = a group of pods with one job · terminal = the entrance hall inside the rock · ETFE = the light see-through plastic skin · medevac = rescue of a sick person, usually by helicopter.)

## 1. The four voids — from empty to useful

Before, the four voids were just empty frames. Today we gave each one a real job, so they help run the whole settlement.

| Void | Before | Now | Why it is better |
| --- | --- | --- | --- |
| V1 | empty frame | Holds wind turbines (makes electricity) and a place to watch the sky and aurora | The strong Arctic wind now makes power instead of only being a danger |
| V2 | empty frame | Has solar panels on its frame; also the open-air deck to watch the northern lights | Catches the long summer sun for electricity |
| V3 | empty frame | Holds the energy store (hydrogen + heat tanks) and catches rain and snow for water | Saves summer power for the dark winter, and gives the settlement its water |
| V4 | empty frame | Open sky over the main square, plus a small deck for drones | Light for the square, and a way to receive deliveries/rescue from the air |

In one line: the voids catch wind, sun and rain from the sky, and the machines that clean and store it are hidden inside the Logistics cluster.

## 2. Power and water — now solved

| Question | Before | Now |
| --- | --- | --- |
| Where does the electricity come from? | Not clearly answered | Wind (V1) + sun (V2) + tidal power from the sea whirlpool → saved as hydrogen + heat for winter (V3) |
| Where does the water come from? | Not answered | Snow and rain caught at the voids, then cleaned inside Logistics |

Why it matters: a building that touches nothing on the ground must make its own power and water. Now it does. This is a strong point for the exam.

## 3. The helicopter pad (helipad) — now safe and fast

The prof and you asked: the helipad should be near the lift and the emergency room. Here is the change.

| Part | Before | Now | Why |
| --- | --- | --- | --- |
| Where the helicopter lands | “somewhere on the summit” | On the solid rock summit, right above the terminal | You cannot land a helicopter safely on the moving, cabled village in strong wind — rock is safe |
| The lift | not clear | One fast lift joins the helipad ↔ terminal ↔ hospital (ER) on one straight line | Rescue becomes short and quick |
| The hospital (ER) | far inside the settlement | Moved next to the Ascent Core, right by the lift | A sick person reaches the helicopter fast |
| The village itself | no air access | A small drone deck (V4) for deliveries and emergency drops | Air help without landing a real helicopter on the structure |

## 4. The cluster plan — now shows the inside

| Before | Now |
| --- | --- |
| Only coloured zones (Research, Logistics, Habitat, Eco) | Each pod is drawn with its job written inside (wet lab, ER, cabins, kitchen, dining, sanctuary, and so on) |
| Bridges not clear | Bridges drawn: grey ones go to the Civic Heart in the middle, red ones make a ring around the outside so every cluster has two ways out |
| No lifts shown | A vertical core (lift + stairs) marked in each cluster |
| Helipad not linked | The helipad → terminal → Civic Heart line is shown |

Why: the prof wanted to see the whole internal function — now it is all on one plan.

## 5. The weather — now a good thing, not only a danger

Before, the design only tried to survive the weather. Now the weather is part of the fun.

| Weather | New space we use it for |
| --- | --- |
| Northern lights (winter) | A warm outdoor pool and sauna to watch them; glass roofs |
| Midnight sun (summer, sun never sets) | Sun decks and a 24-hour garden |
| Dark winter | A warm fire lounge and special light for good mood |
| Storms | A safe glass room to watch the storm — and you can feel the building gently sway (a special idea) |
| The Moskstraumen whirlpool (next to the site) | Watch it, get tidal power from it, and study it |
| Cold sea | The ritual: hot sauna → jump in the cold fjord → rest under the northern lights |

## 6. Going outside — new, and leaves no marks

This is completely new today. Four outdoor activities, all seasonal (only when the snow/season allows) and zero-trace (nothing is built on the ground). Everything starts from the terminal (land) and the pier (sea), and the gear is kept in a Base Camp inside the rock.

| Activity | What it is |
| --- | --- |
| Dog-sledding & ski trails | On the snow, from the terminal |
| Kayak / boat trips | To the whirlpool and to see whales in winter |
| Ice-climbing | On the granite cliffs |
| Field-study & studio points | Small outdoor spots to study weather, rock and tides, and a place for the artists |

One honest note: snowmobiles are mostly not allowed in Norway and the nature is protected — so we keep the fun for dog-sled, ski and kayak. This keeps the “leave no trace” idea strong.

## 7. The pods and light — new architectural detail

The prof wanted interesting architecture inside some pods, and to see how they use light and materials. We designed three special pods, each with a different light idea.

| Pod | Light idea | What is special inside | Materials |
| --- | --- | --- | --- |
| Assembly Hall (Civic) | BRIGHT | A tall, triple-height hall with a stepped timber seating; light comes straight down from a top window (oculus) | Warm timber + glowing ETFE skin |
| Sanctuary (the “believe” area) | DARK & calm | A quiet room; only one shaft of sky/aurora comes down; a still pool reflects it | Stone, dark timber, soft felt (for silence) |
| Living pod (cabins) | CONTROLLED | Bright lounge in the middle; private cabins above with an aurora nook; blinds to block the summer sun, warm light for the dark winter | Warm timber + textile |

The rule for all pods: in summer the sun never sets, so pods must be able to go dark for sleep; in winter it is almost always night, so pods must catch the low sun and aurora and give warm light.

## 8. The boards (tavole) — from 3 to 7, and clearer

| Before | Now |
| --- | --- |
| 3 boards | 7 boards, telling the story in order |
| Wrong size assumed (landscape) | Correct 900 × 900 square size |
| No board for “how you arrive” | Board 2 now explains arrival + how you go up and down (with a plain-words helper) |
| Weak “life” board | Board 7 is now “Life, Weather & the Outdoors” |
| Data shown as messy text | One clean table style for all 10 tables |

## 9. What is still left to do

| Task | Note |
| --- | --- |
| Draw the inside of each pod, floor by floor | We have the side-views (sections); the top-views (plans) are next |
| Add more special pods | A lab and a wellness (sauna) pod |
| Build one real board at full size | Board 2 is the easiest to start |
| Fill the 10 tables with real numbers | Planned, not filled yet |
| Update the Mongolian paper | Still the old version |